Alaska North Slope Wireline ML Classification Methods Draft

Unclassified scaffold for future approved well-log and core data: variables to equations to staged classification to ML outputs

Boundary: This document is a public-source planning and methods scaffold. It contains no classified or restricted well logs, core measurements, identifiers, populated results, or derived sensitive outputs. Future approved data must remain in the authorized runtime environment.

# Abstract

This draft reframes the Alaska North Slope gas hydrate project as a physics-constrained wireline machine-learning workflow. The purpose is not to restate general gas hydrate geology, but to define how future approved well logs and core observations will be converted into interpretable features, staged hydrate evidence, uncertainty labels, producibility screens, and presentation-ready results. The working hypothesis is that hydrate classification requires six separable decisions: data quality, pressure-temperature admissibility, reservoir quality, multi-log phase evidence, competing rock-type explanations, and producibility risk. The scaffold therefore treats every sweet spot as conditional: a variable range is useful only when the supporting curves, rock type, overburden/stress context, and calibration evidence agree.

# 1. Project Direction

The earlier manuscript is best treated as a source accumulation and scientific synthesis. This new direction is a methods paper and dashboard specification for a future real-data project. Its central chain is: measured curves -> derived equations -> physical variables -> staged interpretation gates -> ML features -> interval classifications -> results and discussion.

Primary task: classify hydrate-supportive, gas-supportive, water-bearing, non-reservoir, and expert-review intervals from approved wireline and core data.

Secondary task: rank candidate sweet spots without confusing hydrate detectability, hydrate saturation, and producibility.

Scientific control: every range must carry its physical reason, competing explanations, and required supporting evidence.

ML control: features should be grouped by well and geology so validation does not leak depth-adjacent samples between training and testing.

# 2. Measurement Families and Runtime Inputs

| Family | Measured inputs | Derived features | Decision role |
| --- | --- | --- | --- |
| Lithology | GR, optional mineralogy/core | Vsh, clean-sand flag, rock-type screen | Defines whether a positive response is in plausible reservoir rock. |
| Electrical | Rt, Rw/calibration if available | Archie hydrate proxy | Hydrate can raise resistivity by reducing connected brine pathways, but gas, ice, coal, and low porosity can also raise Rt. |
| Density/porosity | RHOB, NPHI, matrix/fluid assumptions | Density porosity, storage capacity, density QC | Separates pore-volume capacity from hydrate occupancy. |
| Sonic/elastic | DT, DTS, RHOB | Vp, Vs, Vp/Vs, G, K, lambda-rho, mu-rho, Poisson ratio, Young's modulus | Tests whether high Rt is accompanied by hydrate/ice/competent-rock stiffness or gas-like softening. |
| NMR/core | NMR porosity, core porosity/saturation, pressure-core quality | NMR-density hydrate proxy, confidence weights | Preferred calibration path where available; preserves depth-match uncertainty. |
| Thermal/stress | Depth, temperature gradient, pressure, overburden gradient | GHSZ admissibility, vertical stress, effective stress | Hydrate must be stable, and stress changes velocity/porosity interpretation. |

# 3. Equation Library

The equation library below is the minimum mathematical scaffold to represent the future wellbore workflow. The equations are not final field calibrations; they are traceable transforms that define what the dashboard and model should calculate once approved data are available.

| Equation group | Equation | Feature | How it enters classification |
| --- | --- | --- | --- |
| Lithology / reservoir quality | Vsh = clip((GR - GR_clean) / (GR_shale - GR_clean), 0, 1) | Shale volume proxy | Hydrate evidence is downgraded in shale-rich intervals unless core/facies evidence supports it. |
| Density porosity | phi_D = (rho_ma - RHOB) / (rho_ma - rho_f) | Storage-capacity porosity | Good porosity raises reservoir-quality score but does not create a hydrate label. |
| Sonic velocity | Vp = 304.8 / DT; Vs = 304.8 / DTS | Compressional and shear velocity | Hydrate-supportive Rt requires elastic support to avoid gas/low-porosity false positives. |
| Elastic moduli | G = rho * Vs^2; K = rho * (Vp^2 - 4Vs^2/3) | Shear modulus and bulk modulus | Higher shear modulus supports hydrate/ice/competent-rock review; low stiffness with high Rt flags gas or ambiguity. |
| Lambda-rho / mu-rho | lambda-rho = rho * (Vp^2 - 2Vs^2); mu-rho = rho * Vs^2 | Fluid-sensitive and rigidity-sensitive crossplot terms | Used as a crossplot lane; overlap with shale, ice, and carbonates remains explicit. |
| Poisson / Young's modulus | nu = (Vp^2 - 2Vs^2) / (2(Vp^2 - Vs^2)); E = 2G(1 + nu) | Mechanical behavior features | Not a hydrate detector; supports rock-type and producibility interpretation. |
| Pressure-temperature admissibility | T(z) = T0 + GT*z; Pp(z) = rho_w*g*z | Working GHSZ screen | Outside/marginal stability prevents automatic hydrate promotion even if logs look interesting. |
| Overburden / effective stress | sigma_v = integral(rho_b*g dz); sigma_eff = sigma_v - alpha*Pp | Vertical and effective stress | Velocity/stiffness anomalies are reviewed against stress and rock type before hydrate labeling. |
| Saturation proxy | Sh_Archie = 1 - (a*Rw / (phi^m * Rt))^(1/n) | Electrical hydrate proxy | Supplementary only; clay, salinity, gas, low porosity, and ice must be reviewed. |
| NMR-density saturation proxy | Sh_NMRD = (phi_D - phi_NMR) / phi_D | Preferred hydrate proxy where NMR exists | Preferred saturation proxy, but depth matching, shale effects, and tool QC remain visible. |
| Permeability / producibility risk | k_rel_proxy = (1 - Sh)^n | Permeability retention proxy | Separates hydrate occurrence/saturation from production sweet-spot ranking. |

# 4. Why Sweet Spots Are Conditional

A sweet spot is not a single threshold. For this project, a sweet spot is an interval that passes a staged set of physical screens: admissible pressure-temperature conditions, reservoir-quality rock, multi-log hydrate evidence, limited competing explanations, and enough retained flow capacity to justify production-oriented review. The same resistivity or velocity value can mean different things in clean sand, shaly sand, permafrost/ice-bearing sediment, coal, carbonate, or a washed-out borehole.

| Variable | Working tendency | Why useful | Competing explanations | Required support |
| --- | --- | --- | --- | --- |
| GR | Lower GR supports a cleaner sand screen; use local clean/shale anchors. | GR helps distinguish reservoir-grade sand from shale-rich intervals before phase interpretation. | Bentonitic shale, radioactive sand, coal, and local facies variation. | RHOB/porosity, borehole QC, depositional context, and multi-log phase evidence. |
| Rt | Hydrate sand working anchor about 10-100+; water sand about 1-5. | Pore-filling hydrate can reduce conductive water connectivity and increase resistivity. | Gas, low porosity, clay effects, carbonate, coal/lignite, and ice-bearing sediment. | Sonic, density/porosity, lithology, NMR where available, and P-T context. |
| Porosity | Hydrate sand working anchor about 0.20-0.35; interpret capacity separately from occupancy. | Porosity describes storage capacity and supports density/NMR saturation transforms. | Shale correction, matrix choice, gas effect, washout, and thin-bed averaging. | GR, RHOB QC, NMR where available, and local calibration. |
| Vp | Hydrate sand working anchor about 2.5-4.0; gas sand about 1.5-2.5. | Hydrate frame stiffening can increase compressional velocity; gas tends to soften the response. | Stress, cementation, ice-bearing sediment, carbonate, and competent clastics. | Vs, RHOB, GR/lithology, stress context, and P-T screen. |
| Vs | Hydrate sand working anchor about 1.0-2.5; gas sand about 0.3-1.0. | Shear response helps identify rigidity increase and separate hydrate from gas-related compressibility. | Stress, ice-bearing sediment, competent lithology, and missing or unreliable DTS. | Vp, density, GR/lithology, tool QC, and depth context. |
| lambda-rho | Hydrate sand working anchor about 12-55 in Table A.2; end-member crossplots require local calibration. | Lambda-rho helps characterize compressibility and fluid-sensitive elastic behavior. | Gas, shale, carbonates, stress, and mixed intervals. | mu-rho, Vp/Vs, GR, RHOB, and local core/log review. |
| mu-rho | Hydrate sand working anchor about 10-55; gas sand about 1-10. | Rigidity increase is a useful hydrate-supportive tendency when shear data are reliable. | Ice-bearing sediment, shale, carbonates, competent sand, and stress. | lambda-rho, Vp/Vs, GR/lithology, depth relative to permafrost, and P-T context. |
| NMR-density separation | Positive density-minus-NMR porosity separation supports a mobile-fluid-deficit screen. | The manuscript prefers NMR-density saturation where NMR exists. | Tool quality, depth mismatch, shale response, and matrix assumptions. | Density QC, local depth alignment, GR/lithology, and core calibration. |
| Temperature | Use local T(z); North Slope example anchor starts near -10 deg C with 2.0-4.0 deg C/100 m gradients. | Temperature is required for probabilistic stability admissibility. | Local thermal anomalies, uncertain gradient, and non-equilibrium behavior. | Pressure, depth reference, permafrost depth, and local thermal calibration. |
| Pressure | Use local P(z); first-pass hydrostatic anchor about 9.795 kPa/m. | Pressure combines with temperature to define stability admissibility. | Pressure anomalies, compositional effects, local stress, and uncertain depth reference. | Temperature, depth reference, local pressure observations, and uncertainty bounds. |

# 5. Overburden, Pressure, and Rock-Type Effects

Overburden and pore pressure affect the interpretation because increasing effective stress can compact sediments, reduce porosity, and increase velocity even without hydrate. Conversely, high pore pressure can reduce effective stress and alter mechanical response. The dashboard therefore calculates stress context as a review variable, not as a hydrate detector. Rock type is treated the same way: it changes the meaning of each curve and must be attached to every classification lane.

| Rock/context | Why it matters | False-positive risk | Model/dashboard response |
| --- | --- | --- | --- |
| Clean sand | Provides pore volume and likely permeability for hydrate occupancy. | Can be water-bearing with no charge or hydrate occupancy. | Eligible for reservoir-quality score and phase-evidence review. |
| Shaly or bentonitic sand | May host thin or mixed hydrate-bearing layers. | Clay conductivity, bound water, and radioactive minerals distort GR/Rt/porosity transforms. | Downgrade confidence and require NMR/core/facies support. |
| Ice-bearing sediment / permafrost | Can be stiff and resistive in hydrate stability settings. | Ice can overlap hydrate in Vs, mu-rho, and resistivity behavior. | Require depth/permafrost and P-T context before hydrate promotion. |
| Free-gas sand | Can share high resistivity with hydrate-bearing intervals. | Gas often lowers Vp and weakens lambda-rho compared with hydrate. | High Rt plus low Vp/elastic weakness routes to gas-supportive or expert-review lanes. |
| Coal/lignite or carbonate/competent rock | Important regional lithologies and mechanical end members. | Can create high/low density, high resistivity, or high stiffness unrelated to hydrate. | Treat as competing lithology until local log/core evidence resolves it. |

# 6. Staged Classification Workflow

The classifier should act like a disciplined interpreter. It should not jump from a curve value to a label. It should carry an interval through gates, keep uncertainty visible, and preserve good-sand/no-hydrate and expert-review outcomes as valid scientific results.

| Stage | Question | Primary variables | Pass logic |
| --- | --- | --- | --- |
| 1. QC gate | Are borehole conditions and curve coverage good enough? | Caliper, missing DTS/NMR, curve depth alignment | Bad-hole intervals remain review-only until density/sonic reliability is defensible. |
| 2. Stability gate | Can hydrate physically exist at this depth? | Temperature, pressure, depth, local gradient | Inside working GHSZ permits evaluation; it does not prove hydrate. |
| 3. Rock-quality gate | Is there enough clean, connected reservoir volume? | GR/Vsh, density porosity, rock type, facies | Clean porous sand is a candidate host; shale/coal/carbonate require separate handling. |
| 4. Phase-evidence gate | Do electrical, sonic, density, and NMR responses agree? | Rt, Vp, Vs, lambda-rho, mu-rho, NMR-density proxy | Hydrate is promoted only when high Rt is supported by stiffness and saturation evidence. |
| 5. Competing-explanation gate | Could the same signal be gas, ice, shale, coal, carbonate, or stress? | Vp/Vs, mu-rho, Vsh, effective stress, permafrost/depth context | Ambiguous intervals stay in expert review rather than being forced into hydrate/non-hydrate. |
| 6. Producibility ranking | If hydrate is present, is it likely to flow/respond well? | Porosity, permeability proxy, saturation proxy, stress, continuity | Moderate saturation with retained flow can outrank maximum saturation for production review. |

# 7. Machine-Learning Design

The ML system should begin with interpretable baselines and only move toward more flexible models after grouped validation is stable. The first defensible model ladder is: rule-based baseline, regularized logistic regression or calibrated random forest, gradient-boosted trees, and then sequence-aware or interval-aware models if the data volume and labels justify them. The model should output probabilities and reasons, not just class labels.

Features: raw curves, derived equations, QC flags, rock-type screen, pressure-temperature context, overburden/effective stress, and interval statistics.

Labels: hydrate-supportive, gas-supportive, water sand, good sand/no hydrate, non-reservoir, and expert review.

Validation: split by well or fault/reservoir compartment, not by random depth sample.

Calibration: compare probability calibration, confusion matrix, feature importance, and failure cases against core-calibrated intervals.

Outputs: interval table, depth-panel callouts, cross-well variable ranges, sweet-spot lanes, uncertainty flags, and slide-ready figures.

# 8. Results and Discussion Template

When approved data are available, the results section should be written around decisions rather than around raw plots. Each well or reservoir interval should report the staged screen outcome, the equations that drove the classification, the evidence against competing explanations, and the uncertainty remaining after core calibration.

| Result item | Output | Purpose |
| --- | --- | --- |
| Figure 1 | Well-log panel with shaded candidate intervals | Shows where Rt, Vp/Vs, mu-rho, porosity, and NMR-density evidence overlap. |
| Figure 2 | Equation-derived feature crossplots | Shows hydrate/gas/water/review separation in lambda-rho, mu-rho, Vp/Vs, and saturation-proxy space. |
| Figure 3 | Pressure-temperature and stress panel | Explains whether the interval is admissible and whether stress may explain velocity/stiffness. |
| Table 1 | Interval classification table | Reports gate outcomes, phase evidence, saturation proxy, producibility lane, and uncertainty flags. |
| Table 2 | Core-to-log calibration table | Reports depth-match uncertainty, pressure-core quality, measured saturation/porosity, and confidence weights. |
| Discussion | Failure modes and missing evidence | Explains intervals that failed because of gas, ice, shale, coal, washout, stress, or missing NMR/DTS. |

# 9. Current Evidence Anchors and Missing Pieces

The organized source library is strong enough for a scaffold. The remaining gap is not the absence of equations; it is field calibration. The eventual approved-data workflow still needs local clean-sand/shale anchors, matrix-density assumptions, water salinity/resistivity, temperature and pressure calibration, core depth matching, and well-grouped validation labels.

Mount Elbert public studies support multi-log saturation comparison using NMR, P- and S-wave velocity, resistivity, and core context.

Hydrate-01 public work supports acoustic/resistivity/NMR comparison and shows high hydrate occupancy in target reservoir sands.

Eileen Trend public work supports the idea that structural-stratigraphic segmentation and partial fill must constrain interpretation.

North Slope reservoir-quality and overburden sources support treating rock type and stress as contextual controls rather than hydrate labels.

# References and Source Anchors

Lee, M.W., and Collett, T.S. (2011). In-situ gas hydrate saturation estimated from various well logs at the Mount Elbert Gas Hydrate Stratigraphic Test Well, Alaska North Slope. Marine and Petroleum Geology, 28(2), 439-449. DOI: 10.1016/j.marpetgeo.2009.06.007.

Haines, S.S., Collett, T., Yoneda, J., Shimoda, N., Boswell, R., and Okinaka, N. (2022). Gas hydrate saturation estimates, gas hydrate occurrence, and reservoir characteristics based on well log data from the Hydrate-01 stratigraphic test well, Alaska North Slope. Energy & Fuels, 36(6), 3040-3050. DOI: 10.1021/acs.energyfuels.1c04100.

Haines, S.S., Collett, T., Boswell, R., Lim, T., Okinaka, N., Suzuki, K., and Fujimoto, A. USGS summary page: Gas hydrate saturation estimation from acoustic log data in the 2018 Alaska North Slope Hydrate-01 stratigraphic test well.

Zyrianova, M., Collett, T., and Boswell, R. (2024). Characterization of the structural-stratigraphic and reservoir controls on the occurrence of gas hydrates in the Eileen Gas Hydrate Trend, Alaska North Slope.

NETL / DOE Mount Elbert project page. The Alaska North Slope Stratigraphic Test Well: public summary of Mount Elbert well-log data types.

Organized local source library: docs/source_library_index/source_index.md and source_manifest.csv. Use these as a citation checklist, not as final provenance.

# Appendix A. Sweet-Spot Review Lanes

These lanes are designed for review and presentation. They should not be treated as universal field thresholds. The real-data project must recalibrate them against local logs, core observations, reservoir compartment, and uncertainty.

| Planning band | Proxy range | Use | Required confirmation |
| --- | --- | --- | --- |
| Admissibility only | Any | Confirm that the interval is inside a probabilistic GHSZ screen. | Reservoir quality, multi-log phase evidence, and local calibration. |
| Occupancy / expert review | < 0.35 | Preserve low-to-moderate occupancy, thin-bed averaging, and ambiguous responses for review. | Vertical-resolution review, NMR where available, sonic, Rt, and core depth match. |
| Candidate sweet-spot review lane | 0.35-0.60 | Flag a moderate-saturation interval for production-oriented review when reservoir quality and QC remain favorable. | Connected pore volume, permeability retention, pressure communication, core calibration, and geomechanics. |
| High-saturation hydrate-supportive lane | 0.60-0.80 | Flag strong hydrate-supportive evidence while retaining a separate flow-risk review. | NMR-density preferred, sonic and Rt agreement, reservoir continuity, and producibility review. |
| Very-high-saturation resource lane | > 0.80 | Flag high resource density and detectability, not an automatic best production target. | Permeability-collapse risk, pressure communication, mechanical response, and local core calibration. |