PUBLIC-SOURCE RESEARCH DRAFT

Physics-Constrained Wireline Machine Learning for Gas Hydrate Detection and Producibility Screening on the Alaska North Slope

Research-paper-style scaffold built from the manuscript, organized source library, and public-source atlas

Boundary: This document is a public-source planning draft. It contains no classified or restricted well-log data, no approved-runtime identifiers, and no populated sensitive results. Future approved inputs and outputs must remain inside the authorized DOE environment.

# Abstract

Gas hydrate interpretation on the Alaska North Slope requires more than locating intervals inside a pressure-temperature stability field. Hydrate occurrence depends on the coupled overlap of stability, gas charge, reservoir quality, migration access, trapping geometry, water availability, and geologic timing. This draft proposes a physics-constrained wireline machine-learning workflow that converts approved log measurements into auditable interval-scale features, staged labels, uncertainty flags, and producibility ranks. The workflow separates hydrate admissibility, reservoir quality, phase classification, saturation estimation, structural localization, and producibility screening so that high resistivity, favorable stability, or good reservoir quality cannot become shortcut labels.

# 1. Introduction

The central interpretive problem is that the same log response can reflect multiple physical states. High resistivity can indicate hydrate, gas, low porosity, competent lithology, clay effects, carbonate, coal, or ice-bearing sediment. Elevated velocity can reflect hydrate frame stiffening, compaction, cementation, stress, or competent lithology. A research-grade workflow must therefore preserve competing explanations and require multi-log agreement before assigning hydrate-supportive meaning.

The manuscript frames the project as a coupled system: environment -> tectonics -> deposition -> reservoir -> physics -> logs -> interpretation -> ML -> exploitation. The research paper structure below keeps that chain visible while converting it into measurable features and equations.

# 2. Organized Source Base

Candidate sources were organized into a local source library under `Gas hydrates unclassified info/source_library`. The library is a planning aid and should still be reviewed for formal provenance before final citation formatting.

| Source group | Planning role | Examples |
| --- | --- | --- |
| Manuscript synthesis | Preserves project-specific system logic, feature blocks, equations, and failure modes. | Manuscript North Slope draft; TRUE MASTER document; rewritten synthesis |
| USGS and field reports | Provides public field context for Mount Elbert, permafrost-associated hydrates, and Arctic resource framing. | sir2012-5054 summary/methods/chapters; sir2012-5147 Arctic report; Mount Elbert/permafrost report |
| Alaska geology and reservoir quality | Constrains North Slope tectonics, Nanushuk/Torok reservoir architecture, overburden, and basin context. | Nanushuk reservoir quality; North Slope geology/hydrocarbon resources; overburden framework |
| Wireline equations and ranges | Supplies equation maps, screening ranges, lambda-density, P-T, and overburden relationships. | hydrate_wireline_equation_map; lambda_density_pt_overburden_ranges; property range suite |
| Geomechanics, productivity, and ML | Supports permeability retention, load-bearing hydrate, saturation vs productivity, and model-stage logic. | Geomechanical controls; productivity ML framework; final hydrate documents |
| Maps and public data notes | Supports public-source geographic context and atlas figures. | North Slope Data Layer Map; public file manifest; directional survey notes |

# 3. Conceptual Model

Hydrate should be modeled as a staged outcome rather than a single class. The first stage asks whether hydrate is physically admissible under local pressure-temperature conditions. The second stage asks whether the interval is reservoir-grade. The third stage asks whether multi-log evidence supports hydrate, gas, water, mixed phase, non-reservoir lithology, or expert review. Later stages add charge/localization context and producibility ranking.

Stability is necessary but not sufficient.

Reservoir quality is necessary but not sufficient.

High resistivity is evidence, not a hydrate label.

Good sand with no hydrate is a valid class, not a model failure.

Occurrence, saturation, and producibility must remain separate outputs.

# 4. Methods: Runtime Data and Feature Families

The approved runtime workflow should load LAS or CSV well logs locally inside the authorized environment. Curve aliases should map well-specific mnemonics into stable feature families. The public dashboard can demonstrate the workflow only with synthetic data.

| Feature family | Required or optional inputs | Research use | Failure control |
| --- | --- | --- | --- |
| Lithology and reservoir quality | GR, RHOB, porosity, NMR where available | Screen clean sand, shale volume, storage capacity, and net-to-gross proxy. | Clay-rich, bentonitic, coal/lignite, carbonate, thin-bed, and bad-hole flags. |
| Electrical and saturation | Rt, porosity, Rw assumption, Archie parameters, NMR-density separation | Estimate hydrate-supportive saturation proxies without treating Rt as a direct label. | Salinity, gas, low porosity, clay, invasion, and missing-NMR flags. |
| Elastic discrimination | DT/DTC, DTS, RHOB | Compute Vp, Vs, Vp/Vs, impedance, K, G, lambda-rho, and mu-rho. | Stress, cementation, competent lithology, ice-bearing sediment, and missing-DTS flags. |
| Stability context | Depth, pressure, temperature, permafrost/GHSZ assumptions | Define hydrate admissibility and distance to stability boundary. | Probabilistic screen only; never positive hydrate label. |
| Core calibration | Approved core depth, pressure-core quality, interpreted phase, porosity, saturation, disturbance | Attach confidence weights and depth-match uncertainty windows. | Core disturbance, pressure loss, depth mismatch, and missing interval flags. |
| Producibility | Saturation proxy, porosity, Vsh, stress, permeability proxy, continuity | Rank retained flow and practical target quality separately from occurrence. | High saturation can reduce permeability and pressure communication. |

# 5. Equation Catalog for the Scaffold

The following equations are implementation scaffolds derived from the manuscript equation catalogs and wireline source notes. They should be calibrated to approved local units, tool conventions, core observations, and mentor-approved assumptions before operational use.

| ID | Equation | Form | Use | Manuscript/source anchor |
| --- | --- | --- | --- | --- |
| E1 | Compressional velocity | Vp = 1 / DT | Convert sonic slowness to velocity; include unit conversion when DT is in us/ft. | Tables 7.A4, 8.A3, B.1 |
| E2 | Shear velocity | Vs = 1 / DTS | Convert shear slowness to velocity; missing DTS routes to reduced evidence. | Tables 7.A4, 12.A7, B.1 |
| E3 | Acoustic impedance | AI = rho_b * Vp | Elastic evidence and BSR/log translation. | Tables 7.A1, 7.A4 |
| E4 | Shear impedance | SI = rho_b * Vs | Frame rigidity and hydrate/gas discrimination. | Tables 7.A4, 12.A7 |
| E5 | Shear modulus | G = rho_b * Vs^2 | Hydrate-supportive rigidity feature. | Tables 7.A4, A.2, B.1 |
| E6 | Bulk modulus | K = rho_b * (Vp^2 - 4Vs^2/3) | Compressibility-sensitive phase feature. | Tables 7.A4, A.2, B.1 |
| E7 | Lame lambda | lambda = K - 2G/3 | Fluid/compressibility axis for lambda-rho crossplots. | Tables 7.A3, A.3 |
| E8 | Lambda-rho | lambda_rho = lambda * rho_b | Crossplot feature for hydrate, gas, water, and lithology separation. | Tables 7.A3, A.2, A.3 |
| E9 | Mu-rho | mu_rho = G * rho_b | Shear-rigidity feature; hydrate tends toward higher values when DTS is reliable. | Tables 7.A3, A.2, A.3 |
| E10 | Density porosity | phi_D = (rho_ma - rho_b) / (rho_ma - rho_f) | Storage capacity and saturation input. | Tables 8.A7, 12.A6, B.1 |
| E11 | Gamma-ray shale volume | Vsh = (GR - GR_clean) / (GR_shale - GR_clean) | Reservoir-quality filter before phase interpretation. | Tables 8.A7, 12.A6 |
| E12 | Archie water saturation | Sw = (a * Rw / (phi^m * Rt))^(1/n) | Electrical saturation cross-check. | Tables 8.A6, 12.A5, B.1 |
| E13 | Archie hydrate saturation | Sh_Archie = 1 - Sw | Supplementary hydrate-saturation proxy with uncertainty flags. | Tables 8.A6, 12.A5 |
| E14 | NMR-density saturation | Sh_NMRDEN = (phi_D - phi_NMR) / phi_D | Preferred mobile-fluid-deficit proxy where NMR exists. | Tables 8.A7, 12.A6 |
| E15 | Temperature profile | T(z) = T0 + G_T * z | First-pass local stability context. | Tables 6.A1, A.4 |
| E16 | Hydrostatic pressure profile | P(z) = P0 + 0.009795 * z_MPa_per_m | First-pass pressure context; use local pressure where available. | Tables 6.A1, A.4 |
| E17 | Effective stress | sigma_eff = sigma_v - alpha * Pp | Stress correction for velocity and stiffness interpretation. | Tables 9.A1, 12.A8, B.1 |
| E18 | Permeability retention proxy | k_eff = k0 * (1 - Sh)^n | Producibility penalty as hydrate occupies pore space. | Tables 10.A3, 10.A13, 12.A10 |
| E19 | Volumetric resource screen | GIP = GRV * N/G * phi * Sh * E_gas * C_occ | Planning-level in-place resource screen, not recoverability. | Tables 10.A2, B.1 |

# 6. Variable Range Interpretation

Ranges are screening tendencies and end-member anchors, not universal thresholds. Each range should be displayed in the dashboard with competing explanations and required supporting evidence.

| Variable | Working tendency | Why it matters | Competing explanations and required evidence |
| --- | --- | --- | --- |
| GR | Lower GR | Cleaner sand/reservoir-quality screen | Radioactive sand, coal, local facies, bentonite; require RHOB/porosity and depositional context |
| Rt | Hydrate-supportive sands may show high Rt | Reduced conductive water pathways | Gas, low porosity, carbonate, coal, ice, clay; require sonic/NMR/lithology agreement |
| RHOB | Moderate density reduction may support porosity/storage | Density porosity and possible hydrate occupancy | Washout, gas, coal/lignite, matrix assumption; require caliper and local matrix |
| Porosity | Higher phi supports storage capacity | Reservoir can store hydrate | Good sand can still be water-bearing; require charge and phase evidence |
| Vp/Vs | Use as crossplot tendency | Hydrate, gas, water, ice, and lithology have different elastic trends | Overlapping ranges; require lambda-rho/mu-rho and stress context |
| lambda-rho / mu-rho | Hydrate tends to higher rigidity than gas | Separates frame stiffening from gas softening | Ice, carbonate, shale, stress; use multi-attribute crossplot |
| NMR-density separation | Positive separation supports mobile-fluid deficit | Preferred saturation anchor where NMR exists | Depth mismatch, tool quality, shale effects; attach confidence flag |
| Temperature/pressure/GHSZ | Stable/marginal/outside screen | Hydrate admissibility | Not occurrence; charge and reservoir are required |

# 7. Machine-Learning Design

Use grouped splits by well to prevent depth-sample leakage.

Begin with rule and interpretable baselines before tree ensembles or sequence models.

Train or report staged outputs rather than collapsing all decisions into one hydrate/no-hydrate class.

Keep an explicit expert-review or abstain state for missing tools, contradictory evidence, and poor calibration.

Report probability calibration and failure-mode review, not only accuracy.

# 8. Planned Dashboard and PowerPoint Outputs

| Output | Research-paper role | PowerPoint role |
| --- | --- | --- |
| Variable Range Explorer | Summarize distribution, missingness, and depth-window behavior by well/location. | Range tables and curve panels. |
| Hydrate Interpretation Range Guide | Explain why each variable response matters and where it fails. | Methods and safeguards slides. |
| Interval Screening Scaffold | Report staged outputs with uncertainty and good-sand/no-hydrate outcomes. | Result panel and interval table. |
| Core Calibration Scaffold | Attach pressure-core quality, disturbance, and depth-match confidence. | Core-to-log calibration figure. |
| Validation Dashboard | Show grouped metrics, calibration, abstention, and error review. | Performance reporting slide. |

# 9. References and Source Index

Formal references should be finalized after reviewing original source metadata. For the current planning draft, use the organized local source library and the manuscript table anchors below.

| Ref | Source | Use in this draft |
| --- | --- | --- |
| R1 | Manuscript North Slope of Alaska Gas hydrates draft.docx | Project synthesis; Sections 3, 6, 7, 8, 12, 13; Appendix A; Appendix B. |
| R2 | USGS Scientific Investigations Report 2012-5054 files | Mount Elbert / Alaska hydrate field context and methods. |
| R3 | sir2012-5147_arctic.pdf | Arctic Alaska petroleum/geologic resource context. |
| R4 | permafrost-mtelbert (1).pdf | Permafrost-associated hydrate / Mount Elbert context. |
| R5 | Controls on reservoir quality of the Nanushuk Formation.pdf | Nanushuk reservoir-quality and burial/porosity context. |
| R6 | GEOLOGICAL EVOLUTION AND HYDROCARBON RESOURCES POTENTIAL IN NORTH SLOPE, ALASKA.pdf | North Slope tectonic and petroleum-system context. |
| R7 | hydrate_wireline_equation_map.docx | Wireline equation mapping and implementation notes. |
| R8 | lambda_density_pt_overburden_ranges.docx | Lambda-density, pressure-temperature, and overburden range notes. |
| R9 | Geomechanical_Controls_Gas_Hydrate_Productivity_NGHP_GOM_Alaska_ML.docx | Geomechanical and productivity ML framework. |
| R10 | North Slope Data Layer Map.pdf and public atlas data dictionary | Public-source regional atlas and map-output context. |

Next citation task: Open the source PDFs and convert this planning source list into formal APA/AGU-style citations with authors, years, titles, report numbers, journals, and URLs/DOIs where available.